# Project #6: Explain IP Subnetting Worksheet

## Section 1 /24 Network

Network Address: 192.168.4.0

Broadcast Address: 192.168.4.255

Available Hosts: 254

First Assignable Host: 192.168.4.1

Last Assignable Host: 192.168.4.254

## Section 2 /25 Subnet 1

Network Address: 192.168.4.0

Broadcast Address: 192.168.4.127

Available Hosts: 126

First Assignable Host: 192.168.4.1

## Section 2 /25 Subnet 2

Network Address: 192.168.4.128

Broadcast Address: 192.168.4.255

Available Hosts: 126

Last Assignable Host: 192.168.4.254

## Section 3 /26 Network

Subnet 1 Network Address: 192.168.4.0

Subnet 2 Network Address: 192.168.4.64

Subnet 3 Network Address: 192.168.4.128

Subnet 4 Network Address: 192.168.4.192

## Section 4 /27 Network (answer these questions for the 5th subnet in the network)

Subnet 5 Network Address: 192.168.4.128

Subnet 5 Broadcast Address: 192.168.4.159

Available Hosts: on Subnet 5: 30

First Assignable Host on Subnet 5: 192.168.4.129

Last Assignable Host on Subnet 5: 192.168.4.158

## Section 5 – Subnetting Scenario

Subnet the given network to meet the given host requirements.

# Network: 192.168.110.0/24

| Department | Hosts | Subnet Size | CIDR | Network | Subnet Mask | Sort Order* |
| --- | --- | --- | --- | --- | --- | --- |
| Sales | 80 | 128 | /25 | 192.168.110.0 | 255.255.255.128 | 1 |
| Advertising | 35 | 64 | /26 | 192.168.110.128 | 255.255.255.192 | 2 |
| Research | 18 | 32 | /27 | 192.168.110.192 | 255.255.255.224 | 3 |
| Marketing | 11 | 16 | /28 | 192.168.110.224 | 255.255.255.240 | 4 |
| Administration | 6 | 8 | /29 | 192.168.110.240 | 255.255.255.248 | 5 |
| Router->Router | 2 | 4 | /30 | 192.168.110.248 | 255.255.255.252 | 6 |
| CEO | 2 | 4 | /30 | 192.168.110.252 | 255.255.255.252 | 7 |

*Sort Order means to rank the network requirement big to small. Subnet in that order and your networks will always work. There are alternate solutions but this is the easiest way to do it and is how your project may be graded.

# Scenarios

## Scenario 1: Network 192.168.4.0/24

Question 1: Why are these computers able to communicate? These computers were able to communicate as they both share the same subnet prefix (/24) which also means that they are on the same local network, that is 192.168.4.0/24. Since they are on the same subnet, they can communicate directly and smoothly without a router.

## Scenario 2

Question 2: Why are these computers not able to communicate? As both computers the subnet mask /26, I have taken note that the network was split into groups of 64. PC0 is in the first subnet (0-63), while the PC1 is in the second subnet (64-127). Learning that they are on the different subnets, they cannot talk or communicate with each other directly through a switch.

Question 3: What could you do to PC0 to enable the computers to communicate? To enable the computers to communicate, I will then change PC0’s IP address to a new or unused IP that is within the same subnet with PC1, or I will change the subnet masks on these two computers back to /24.

## Scenario 3

Question 4: What is the subnet mask for this scenario? The subnet mask for this scenario is 255.255.255.240 as /28 utilizes or uses block size of 16.

Question 5: Can PC0 communicate with PC1? Click or tap here to enter text. Why or Why Not?  Yes, PC0 can communicate with PC1, since /28 uses block size of 16, the Subnet 2 extends from 192.168.4.16 to 192.168.4.31. Therefore, both PC0 and PC1 fall within this self-same subnet range.

Question 6: Can PC2 communicate with PC3? Click or tap here to enter text. Why or Why Not?  No, PC2 cannot communicate with PC3 as PC2 is on the subnet that spans from .64 to .79, while PC3 is on the next subnet which is from .80 to 95. Since they are on the different subnets, the switch could not connect them or bridge them together directly.